Quick Reference (Important parameters)   Aug 15, 2019

[parameters] section

icntl: Basic control option. =0; Normal PHITS calculation, =5; void mode, =8; 2D geometry output , =11; 3D geometry output with [t-3dshow], =14; Volume calculation

maxcas: Number of histories per batch

maxbch: Number of batches

istdev: Control parameter for statistical uncertainty calculation type and restart mode. <0; Restart calculation, =1 or -1; Batch variance, =2 or -2; History variance

[source] section

totfact: Scaling factor

s-type: Source shape type. =1; cylinder, =2; box, =9; sphere and spherical shell

proj: Source particle type

dir: Direction cosine of projectile against z axis. =all; isotropic

e0: Energy of projectile in MeV/u

[material] section

Positive fraction indicates atomic ratio. e.g.  H 2  O 1

Negative fraction indicates mass ratio. e.g. H -2/18  O -16/18

[surface] section

so, sx, sy, sz, s: spherical surface

px, py, pz: plane

cx, cy, cz: cylindrical surface

rpp: rectangular solid

[cell] section

Material ID = -1: Outer region

Material ID = 0: void

Positive density: atomic density in 1024 atoms/cm3

Negative density: mass density in g /cm3

Tally types

[t-track], [t-cross]: calculate fluences in volume or on surface

[t-deposit]: calculate deposition energies

[t-product],[t-yield]: calculate production yields of secondary particles

[t-3dshow]: draw 3D geometry when icntl=11

gshow option: dray 2D geometry by tally with mesh = xyz when icntl=8

Tally sections

mesh: shape of the tally region (xyz, r-z, or reg)

part: particle name (proton, neutron …)

unit: unit of tally output

axis: horizontal axis of tally output (x, y, z, xy, xz, yz, eng …)

x-type: mesh type (x can be replaced by y, z, e, a, t …)

- =1; Give number of groups and mesh points by data

e.g. x-type=1

nx = 10

0  1  2  3  5  10  15  20  30  50  100

- =2,3; Give number of groups, minimum and maximum values. Mesh is divided equally by linear and log scales for 2 and 3, respectively.

e.g. x-type=2

nx = 100

xmin = 0

xmax = 1000

file: output file name

epsout: eps output option. =0; No output, =1; output, =2; output with error bar

angel: ANGEL parameters. e.g. =xmin(***)  xmax(***)  ymin(***)  ymax(***)

Others

infl: inserting other files. infl:{file_name}[n1-n2], where n1 and n2 are first and last line numbers to be inserted, respectively

set: defining user variables. set:ci[value], where i should be between 1 and 99.